Name: Dhruv Jain					 Rollno.: 180020006

Digital Systems (EE 204) Assignment-1

Project Link for all VHDL Files :-

https://github.com/Feetly/Course_Projects/tree/master/11.%20ModelSim%20Projects

Q.1 Write a VHDL code to design an arithmetic and logic unit using logic gates that performs the following three operations:

1. 4-bit addition

2. 4-bit subtraction

3. 4-bit multiplication

Write a test bench that includes 10 input combinations for the above program.

A.1 => Inputs A, B, S. Output R. S = 00 (add), S = 01 (Sub), S = 10 (Multiply), S = 11( Concate)

Code: -

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity full_sub is

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end full_sub;

architecture dataflow of full_sub is

begin

S <=  A XOR B XOR Cin ;

Cout <= ((not A) AND B) OR (Cin AND (not A)) OR (Cin AND B) ;

end dataflow;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity full_adder is

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end full_adder;

architecture dataflow of full_adder is

begin

S <= A XOR B XOR Cin ;

Cout <= (A AND B) OR (Cin AND A) OR (Cin AND B) ;

end dataflow;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity Multiplier is

port(

A,B: in std_logic_vector(3 downto 0);

R: out std_logic_vector(7 downto 0)

);

end entity Multiplier;

architecture Behavioral of Multiplier is

begin

R <= std_logic_vector(unsigned(A) * unsigned(B));

end architecture Behavioral;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity calc is

Port(

A : in std_logic_vector(3 downto 0);

B : in std_logic_vector(3 downto 0);

S : in std_logic_vector(1 downto 0);

R : out std_logic_vector(7 downto 0));

end calc;

architecture Behavioral of calc is

component full_adder

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end component;

component full_sub

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end component;

COMPONENT Multiplier

port(

A , B: in std_logic_vector(3 downto 0);

R: out std_logic_vector(7 downto 0)

);

END COMPONENT;

signal RA : std_logic_vector(7 downto 0) := (others => '0');

signal RS : std_logic_vector(7 downto 0) := (others => '0');

signal RM : std_logic_vector(7 downto 0) := (others => '0');

signal tmp : std_logic := '0';

signal c1,c2,c3: STD_LOGIC;

signal f1,f2,f3: STD_LOGIC;

begin

FS1: full_sub port map( A(0), B(0), tmp,  RS(0), f1);

FS2: full_sub port map( A(1), B(1), f1, RS(1), f2);

FS3: full_sub port map( A(2), B(2), f2, RS(2), f3);

FS4: full_sub port map( A(3), B(3), f3, RS(3), RS(4));

M1 : Multiplier port map(A,B,RM);

FA1: full_adder port map( A(0), B(0), tmp,  RA(0), c1);

FA2: full_adder port map( A(1), B(1), c1, RA(1), c2);

FA3: full_adder port map( A(2), B(2), c2, RA(2), c3);

FA4: full_adder port map( A(3), B(3), c3, RA(3), RA(4));

process (S,RA,RS,RM,A,B)

begin

if S = "00" then

R <= RA;

elsif S = "01"	then

R <= RS;

elsif S = "10"	then

R <= RM;

else

R(0) <= B(0);

R(1) <= B(1);

R(2) <= B(2);

R(3) <= B(3);

R(4) <= A(0);

R(5) <= A(1);

R(6) <= A(2);

R(7) <= A(3);

end if;

end process;

end Behavioral;

Test Banch Code:-

library ieee;

use ieee.std_logic_1164.all;

entity calc_tb is

end calc_tb;

architecture behavior OF calc_tb is

COMPONENT calc

Port(

A : in std_logic_vector(3 downto 0);

B : in std_logic_vector(3 downto 0);

S : in std_logic_vector(1 downto 0);

R : out std_logic_vector(7 downto 0));

END COMPONENT;

signal A_tb : std_logic_vector(3 downto 0) := (others => '0');

signal B_tb : std_logic_vector(3 downto 0) := (others => '0');

signal S_tb : std_logic_vector(1 downto 0) := (others => '0');

signal R_tb : std_logic_vector(7 downto 0);

BEGIN

uut: calc PORT MAP (

A => A_tb,

B => B_tb,

S => S_tb,

R => R_tb

);

process

begin

S_tb <= "00";

A_tb <= "1111";

B_tb <= "1111";

wait for 100 ns;

S_tb <= "01";

A_tb <= "1111";

B_tb <= "1111";

wait for 100 ns;

S_tb <= "10";

A_tb <= "1111";

B_tb <= "1111";

wait for 100 ns;

S_tb <= "11";

A_tb <= "1111";

B_tb <= "1111";

wait for 100 ns;

S_tb <= "00";

A_tb <= "1001";

B_tb <= "1010";

wait for 100 ns;

S_tb <= "01";

A_tb <= "1001";

B_tb <= "1010";

wait for 100 ns;

S_tb <= "10";

A_tb <= "1001";

B_tb <= "1010";

wait for 100 ns;

S_tb <= "11";

A_tb <= "1001";

B_tb <= "1010";

wait for 100 ns;

S_tb <= "00";

A_tb <= "1100";

B_tb <= "0011";

wait for 100 ns;

S_tb <= "01";

A_tb <= "1101";

B_tb <= "0010";

wait for 100 ns;

S_tb <= "10";

A_tb <= "1011";

B_tb <= "1011";

wait for 100 ns;

S_tb <= "11";

A_tb <= "0111";

B_tb <= "1110";

wait for 100 ns;

wait;

end process;

END;

Output:

Q.2 Write a VHDL code generates parity as 1 if the number of 1’s a 4-bit long binary message is even and generates 0 otherwise. Use logic gates to design such parity generator. Also, write a VHDL code designing a circuit for parity checker which consider the 5-bit long binary signal (4-bit message + 1-bit parity) and generates 1 if the number of 1’s in the 5-bit binary signal is even and 0 otherwise.

Motivation: If the parity checker generates ‘1’ that means the message signal has error in it. Assumption: Noise can change only 1-bit of the signal

A.2 =>

Part 1: 4-bit parity checker

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity parity is

port(

A: in std_logic_vector(3 downto 0);

R: out std_logic

);

end entity parity;

architecture Behavioral of parity is

begin

R <= NOT(A(3) XOR A(2) XOR A(1) XOR A(0));

end architecture Behavioral;

Test Bench:

library ieee;

use ieee.std_logic_1164.all;

entity parity_tb is

end parity_tb;

architecture behavior OF parity_tb is

COMPONENT parity

port(

A: in std_logic_vector(3 downto 0);

R: out std_logic

);

END COMPONENT;

signal A_tb : std_logic_vector(3 downto 0) := (others => '0');

signal R_tb : std_logic;

BEGIN

uut: parity PORT MAP (

A => A_tb,

R => R_tb

);

process

begin

A_tb <= "0000";

wait for 100 ns;

A_tb <= "0001";

wait for 100 ns;

A_tb <= "0010";

wait for 100 ns;

A_tb <= "0011";

wait for 100 ns;

A_tb <= "0100";

wait for 100 ns;

A_tb <= "0101";

wait for 100 ns;

A_tb <= "0111";

wait for 100 ns;

A_tb <= "1000";

wait for 100 ns;

A_tb <= "1010";

wait for 100 ns;

A_tb <= "1111";

wait for 100 ns;

wait;

end process;

END;

Output:

Part 2: 5-bit parity adder

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity parity2 is

port(

A: in std_logic_vector(3 downto 0);

R: out std_logic

);

end entity parity2;

architecture Behavioral of parity2 is

signal B : std_logic;

begin

B <= NOT(A(3) XOR A(2) XOR A(1) XOR A(0));

R <= NOT(B XOR A(3) XOR A(2) XOR A(1) XOR A(0));

end architecture Behavioral;

Test Bench:

library ieee;

use ieee.std_logic_1164.all;

entity parity2_tb is

end parity2_tb;

architecture behavior OF parity2_tb is

COMPONENT parity2

port(

A: in std_logic_vector(3 downto 0);

R: out std_logic

);

END COMPONENT;

signal A_tb : std_logic_vector(3 downto 0) := (others => '0');

signal R_tb : std_logic;

BEGIN

uut: parity2 PORT MAP (

A => A_tb,

R => R_tb

);

process

begin

A_tb <= "0000";

wait for 100 ns;

A_tb <= "0001";

wait for 100 ns;

A_tb <= "0010";

wait for 100 ns;

A_tb <= "0011";

wait for 100 ns;

A_tb <= "0100";

wait for 100 ns;

A_tb <= "0101";

wait for 100 ns;

A_tb <= "0111";

wait for 100 ns;

A_tb <= "1000";

wait for 100 ns;

A_tb <= "1010";

wait for 100 ns;

A_tb <= "1111";

wait for 100 ns;

wait;

end process;

END;

Output:

Q.3 Write a VHDL code for a 4X1 MUX. Write a test bench and observe the output. Implement the following function using the 4X1 MUX as component in the code.

𝐹(𝐴,𝐵,𝐶,𝐷) = ∑(1,2,4,7,10,12,14,15)

A.3 =>

Part 1: 4X1 MUX

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity MUX is

Port(

S : in  STD_LOGIC_VECTOR (1 downto 0);

I : in  STD_LOGIC_VECTOR (3 downto 0);

Y : out STD_LOGIC);

end MUX;

architecture Behavioral of MUX is

begin

process (S,I)

begin

if (S <= "00") then

Y <= I(0);

elsif (S <= "01") then

Y <= I(1);

elsif (S <= "10") then

Y <= I(2);

else

Y <= I(3);

end if;

end process;

end Behavioral;

Test Bench:

library ieee;

use ieee.std_logic_1164.all;

entity MUX_tb is

end MUX_tb;

architecture behavior OF MUX_tb is

COMPONENT MUX

Port(

S : in  STD_LOGIC_VECTOR (1 downto 0);

I : in  STD_LOGIC_VECTOR (3 downto 0);

Y : out STD_LOGIC);

END COMPONENT;

signal S_tb : std_logic_vector(1 downto 0) := (others => '0');

signal I_tb : std_logic_vector(3 downto 0) := (others => '0');

signal Y_tb : std_logic;

BEGIN

uut: MUX PORT MAP (

S => S_tb,

I => I_tb,

Y => Y_tb

);

process

begin

S_tb <= "00";

I_tb <= "0000";

wait for 100 ns;

S_tb <= "01";

I_tb <= "0000";

wait for 100 ns;

S_tb <= "10";

I_tb <= "0000";

wait for 100 ns;

S_tb <= "11";

I_tb <= "0000";

wait for 100 ns;

S_tb <= "00";

I_tb <= "1001";

wait for 100 ns;

S_tb <= "01";

I_tb <= "0111";

wait for 100 ns;

S_tb <= "10";

I_tb <= "0001";

wait for 100 ns;

S_tb <= "11";

I_tb <= "1010";

wait for 100 ns;

S_tb <= "11";

I_tb <= "0111";

wait for 100 ns;

S_tb <= "01";

I_tb <= "1111";

wait for 100 ns;

wait;

end process;

END;

Output:

Part 2: Implementing function

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity MUXX is

Port(

S1 : in  STD_LOGIC;

S0 : in  STD_LOGIC;

I3 : in  STD_LOGIC;

I2 : in  STD_LOGIC;

I1 : in  STD_LOGIC;

I0 : in  STD_LOGIC;

Y : out STD_LOGIC);

end MUXX;

architecture Behavioral of MUXXis

begin

process (S1,S0,I3,I2,I1,I0)

begin

if (S1 = '0' AND S0 = '0') then

Y <= I0;

elsif (S1 = '0' AND S0 = '1') then

Y <= I1;

elsif (S1 = '1' AND S0 = '0') then

Y <= I2;

else

Y <= I3;

end if;

end process;

end Behavioral;

library ieee;

use ieee.std_logic_1164.all;

entity Func is

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

C : in STD_LOGIC;

D : in STD_LOGIC;

Y0 : out STD_LOGIC);

end Func;

architecture Behavioral of Func is

component MUXX

Port(

S1 : in  STD_LOGIC;

S0 : in  STD_LOGIC;

I3 : in  STD_LOGIC;

I2 : in  STD_LOGIC;

I1 : in  STD_LOGIC;

I0 : in  STD_LOGIC;

Y : out STD_LOGIC);

end component;

signal I1,I2,I3,I0: std_logic;

begin

I0 <= C xor D;

I1 <= C xnor D;

I2 <= C and (not D);

I3 <= C or (not D);

uut: MUXX port map( A , B, I3 , I2 , I1 , I0 , Y0 );

end Behavioral;

Test Bench:

library ieee;

use ieee.std_logic_1164.all;

entity Func_tb is

end Func_tb;

architecture behavior OF Func_tb is

COMPONENT Func

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

C : in STD_LOGIC;

D : in STD_LOGIC;

Y0 : out STD_LOGIC);

END COMPONENT;

signal A_tb : std_logic := '0';

signal B_tb : std_logic := '0';

signal C_tb : std_logic := '0';

signal D_tb : std_logic := '0';

signal Y0_tb : std_logic;

BEGIN

uut: Func PORT MAP (

A => A_tb,

B => B_tb,

C => C_tb,

D => D_tb,

Y0 => Y0_tb

);

process

begin

A_tb <= '0'; B_tb <= '0'; C_tb <= '0'; D_tb <= '0';

wait for 100 ns;

A_tb <= '0'; B_tb <= '0'; C_tb <= '0'; D_tb <= '1';

wait for 100 ns;

A_tb <= '0'; B_tb <= '0'; C_tb <= '1'; D_tb <= '1';

wait for 100 ns;

A_tb <= '1'; B_tb <= '0'; C_tb <= '0'; D_tb <= '0';

wait for 100 ns;

A_tb <= '0'; B_tb <= '0'; C_tb <= '0'; D_tb <= '1';

wait for 100 ns;

A_tb <= '0'; B_tb <= '1'; C_tb <= '0'; D_tb <= '1';

wait for 100 ns;

A_tb <= '0'; B_tb <= '1'; C_tb <= '1'; D_tb <= '0';

wait for 100 ns;

A_tb <= '1'; B_tb <= '0'; C_tb <= '0'; D_tb <= '1';

wait for 100 ns;

A_tb <= '1'; B_tb <= '0'; C_tb <= '1'; D_tb <= '0';

wait for 100 ns;

A_tb <= '1'; B_tb <= '1'; C_tb <= '1'; D_tb <= '1';

wait for 100 ns;

wait;

end process;

END;

Output:

Q.4 Write a VHDL code for a D-flipflop using behavioural modelling. Use the D-Flipflop as a component to design a 4-bit universal shift register that does the following operation:

A B Operation 1 1 Loads Parallel Data 1 0 Circular Left Shift 0 1 Circular Right Shift

Write a testbench along with a suitable clock input to observe the output.

A.4 =>

Part 1: D FlipFlop

library ieee;

use ieee.std_logic_1164.all;

entity D_FlipFlop is

port(

I: in std_logic;

clk: in std_logic;

rst: in std_logic;

prst: in std_logic;

Y: out std_logic);

end D_FlipFlop;

architecture behavioral of D_FlipFlop is

begin

process(prst,rst,clk,I)

begin

if (rst='1') then

Y <= '0';

elsif (prst='1') then

Y <= '1' ;

elsif(rising_edge(clk)) then

Y <= I;

end if;

end process;

end behavioral;

Test Bench:

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY D_FlipFlop_tb IS

END D_FlipFlop_tb;

ARCHITECTURE behavior OF D_FlipFlop_tb IS

COMPONENT D_FlipFlop

port(

I: in std_logic;

clk: in std_logic;

rst: in std_logic;

prst: in std_logic;

Y: out std_logic);

END COMPONENT;

signal I_tb : std_logic := '0';

signal clk_tb : std_logic := '0';

signal rst_tb : std_logic := '1';

signal prst_tb : std_logic := '0';

signal Y_tb : std_logic;

constant clk_period : time := 20 ns;

signal ctr : integer := 0;

BEGIN

uut: D_FlipFlop PORT MAP (

I => I_tb,

clk => clk_tb,

rst => rst_tb,

prst => prst_tb,

Y => Y_tb

);

clk_process :process

begin

clk_tb <= '0';

wait for clk_period/2;

clk_tb <= '1';

wait for clk_period/2;

ctr <= ctr+20;

if (ctr = 380) then

wait;

end if;

end process;

stim_proc: process

begin

rst_tb <= '1';

prst_tb <= '0';

wait for 100 ns;

rst_tb <= '0';

prst_tb <= '1';

wait for 100 ns;

rst_tb <= '0';

prst_tb <= '0';

I_tb <= '0';

wait for 100 ns;

rst_tb <= '0';

prst_tb <= '0';

I_tb <= '1';

wait for 100 ns;

wait;

end process;

END;

Output:

Part 2: Circular Register

library ieee;

use ieee.std_logic_1164.all;

entity D_FlipFlop is

port(

I: in std_logic;

clk: in std_logic;

rst: in std_logic;

prst: in std_logic;

YD: out std_logic);

end D_FlipFlop;

architecture behavioral of D_FlipFlop is

begin

process(prst,rst,clk,I)

begin

if (rst='1') then

YD <= '0';

elsif (prst='1') then

YD <= '1' ;

elsif(rising_edge(clk)) then

YD <= I;

end if;

end process;

end behavioral;

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity MUX is

Port(

S1 : in  STD_LOGIC;

S0 : in  STD_LOGIC;

I3 : in  STD_LOGIC;

I2 : in  STD_LOGIC;

I1 : in  STD_LOGIC;

I0 : in  STD_LOGIC;

YM : out STD_LOGIC);

end MUX;

architecture Behavioral of MUX is

begin

process (S1,S0,I3,I2,I1,I0)

begin

if (S1 = '0' AND S0 = '0') then

YM <= I0;

elsif (S1 = '0' AND S0 = '1') then

YM <= I1;

elsif (S1 = '1' AND S0 = '0') then

YM <= I2;

else

YM <= I3;

end if;

end process;

end Behavioral;

library ieee;

use ieee.std_logic_1164.all;

entity Cir_Register is

Port(

Inp : in std_logic_vector(3 downto 0);

Q: out std_logic_vector(3 downto 0);

S : in std_logic_vector(1 downto 0);

clck: in std_logic);

end Cir_Register;

architecture Behavioral of Cir_Register is

component D_FlipFlop

port(

I: in std_logic;

clk: in std_logic;

rst: in std_logic;

prst: in std_logic;

YD: out std_logic);

end component;

component MUX

Port(

S1 : in  STD_LOGIC;

S0 : in  STD_LOGIC;

I3 : in  STD_LOGIC;

I2 : in  STD_LOGIC;

I1 : in  STD_LOGIC;

I0 : in  STD_LOGIC;

YM : out STD_LOGIC);

end component;

signal Y : std_logic_vector(3 downto 0) :=  (others => '0');

signal QM : std_logic_vector(3 downto 0) :=  (others => '0');

begin

M3: MUX port map( S(1) , S(0), Inp(3) , QM(2) , QM(0) , '0' , Y(3) );

M2: MUX port map( S(1) , S(0), Inp(2) , QM(1) , QM(3) , '0' , Y(2) );

M1: MUX port map( S(1) , S(0), Inp(1) , QM(0) , QM(2) , '0' , Y(1) );

M0: MUX port map( S(1) , S(0), Inp(0) , QM(3) , QM(1) , '0' , Y(0) );

D3: D_FlipFlop PORT MAP( Y(3) , clck , '0' , '0' , QM(3));

D2: D_FlipFlop PORT MAP( Y(2) , clck , '0' , '0' , QM(2));

D1: D_FlipFlop PORT MAP( Y(1) , clck , '0' , '0' , QM(1));

D0: D_FlipFlop PORT MAP( Y(0) , clck , '0' , '0' , QM(0));

Q <= QM;

end Behavioral;

--------------------------------------------------------------------------------------------------------------------------------------------------

Test Bench:

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY Cir_Register_tb IS

END Cir_Register_tb;

ARCHITECTURE behavior OF Cir_Register_tb IS

COMPONENT Cir_Register

Port(

Inp : in std_logic_vector(3 downto 0);

Q: out std_logic_vector(3 downto 0);

S : in std_logic_vector(1 downto 0);

clck: in std_logic);

END COMPONENT;

signal Inp_tb : std_logic_vector(3 downto 0) :=  (others => '0');

signal S_tb : std_logic_vector(1 downto 0) := (others => '0');

signal Q_tb: std_logic_vector(3 downto 0);

signal clck_tb: std_logic := '0';

constant clk_period : time := 20 ns;

signal ctr : integer := 0;

BEGIN

uut: Cir_Register PORT MAP(

Inp => Inp_tb,

clck => clck_tb,

S => S_tb,

Q => Q_tb

);

clck_process :process

begin

clck_tb <= '0';

wait for clk_period/2;

clck_tb <= '1';

wait for clk_period/2;

ctr <= ctr+20;

if (ctr = 480) then

wait;

end if;

end process;

stim_proc: process

begin

S_tb <= "11";

Inp_tb <= "1000";

wait for 50 ns;

S_tb <= "01";

wait for 100 ns;

S_tb <= "10";

wait for 100 ns;

S_tb <= "11";

Inp_tb <= "1001";

wait for 50 ns;

S_tb <= "01";

wait for 100 ns;

S_tb <= "10";

wait for 100 ns;

wait;

end process;

END;

Output:

Extras:

1. 4-bit Adder

Code

library ieee;

use ieee.std_logic_1164.all;  -- Highlights

entity full_adder is

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end full_adder;

architecture dataflow of full_adder is

begin

S <= A XOR B XOR Cin ;

Cout <= (A AND B) OR (Cin AND A) OR (Cin AND B) ;

end dataflow;

library ieee;

use ieee.std_logic_1164.all;  -- Highlights

entity adder is

Port ( A3 : in STD_LOGIC;

A2 : in STD_LOGIC;

A1 : in STD_LOGIC;

A0 : in STD_LOGIC;

B3 : in STD_LOGIC;

B2 : in STD_LOGIC;

B1 : in STD_LOGIC;

B0 : in STD_LOGIC;

S3 : out STD_LOGIC;

S2 : out STD_LOGIC;

S1 : out STD_LOGIC;

S0 : out STD_LOGIC;

Cout : out STD_LOGIC);

end adder;

architecture Behavioral of adder is

component full_adder

Port ( A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end component;

signal c1,c2,c3: STD_LOGIC;

signal c0 : std_logic := '0';

begin

FA1: full_adder port map( A0, B0, c0, S0, c1);

FA2: full_adder port map( A1, B1, c1, S1, c2);

FA3: full_adder port map( A2, B2, c2, S2, c3);

FA4: full_adder port map( A3, B3, c3, S3, Cout);

end Behavioral;

Test bench:

library ieee;

use ieee.std_logic_1164.all;

entity adder_tb is

end adder_tb;

architecture behavior OF adder_tb is

COMPONENT adder

Port ( A3 : in STD_LOGIC;

A2 : in STD_LOGIC;

A1 : in STD_LOGIC;

A0 : in STD_LOGIC;

B3 : in STD_LOGIC;

B2 : in STD_LOGIC;

B1 : in STD_LOGIC;

B0 : in STD_LOGIC;

S3 : out STD_LOGIC;

S2 : out STD_LOGIC;

S1 : out STD_LOGIC;

S0 : out STD_LOGIC;

Cout : out STD_LOGIC);

END COMPONENT;

signal a3_tb : std_logic := '0';

signal a2_tb : std_logic := '0';

signal a1_tb : std_logic := '0';

signal a0_tb : std_logic := '0';

signal b3_tb : std_logic := '0';

signal b2_tb : std_logic := '0';

signal b1_tb : std_logic := '0';

signal b0_tb : std_logic := '0';

signal s3_tb : std_logic;

signal s2_tb : std_logic;

signal s1_tb : std_logic;

signal s0_tb : std_logic;

signal cout_tb : std_logic;

BEGIN

uut: adder PORT MAP (

A3 => a3_tb,

A2 => a2_tb,

A1 => a1_tb,

A0 => a0_tb,

B3 => b3_tb,

B2 => b2_tb,

B1 => b1_tb,

B0 => b0_tb,

S3 => s3_tb,

S2 => s2_tb,

S1 => s1_tb,

S0 => s0_tb,

Cout => cout_tb

);

process

begin

a3_tb <= '0'; a2_tb <= '0'; a1_tb <= '0'; a0_tb <= '0';

b3_tb <= '0'; b2_tb <= '0'; b1_tb <= '0'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '0'; b2_tb <= '0'; b1_tb <= '0'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '1'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '0'; a1_tb <= '0'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '1'; a1_tb <= '0'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '0'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '0';

b3_tb <= '0'; b2_tb <= '1'; b1_tb <= '1'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '0'; a0_tb <= '0';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '0'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '1'; a1_tb <= '0'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

wait;

end process;

END;

Output:

2. 4-bit Subtractor

Code

library ieee;

use ieee.std_logic_1164.all;  -- Highlights

entity full_sub is

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end full_sub;

architecture dataflow of full_sub is

begin

S <=  A XOR B XOR Cin ;

Cout <= ((not A) AND B) OR (Cin AND (not A)) OR (Cin AND B) ;

end dataflow;

library ieee;

use ieee.std_logic_1164.all;  -- Highlights

entity sub is

Port(

A3 : in STD_LOGIC;

A2 : in STD_LOGIC;

A1 : in STD_LOGIC;

A0 : in STD_LOGIC;

B3 : in STD_LOGIC;

B2 : in STD_LOGIC;

B1 : in STD_LOGIC;

B0 : in STD_LOGIC;

S3 : out STD_LOGIC;

S2 : out STD_LOGIC;

S1 : out STD_LOGIC;

S0 : out STD_LOGIC;

Cout : out STD_LOGIC);

end sub;

architecture Behavioral of sub is

component full_sub

Port(

A : in STD_LOGIC;

B : in STD_LOGIC;

Cin : in STD_LOGIC;

S : out STD_LOGIC;

Cout : out STD_LOGIC);

end component;

signal c1,c2,c3: STD_LOGIC;

signal c0 : std_logic := '0';

begin

FS1: full_sub port map( A0, B0, c0, S0, c1);

FS2: full_sub port map( A1, B1, c1, S1, c2);

FS3: full_sub port map( A2, B2, c2, S2, c3);

FS4: full_sub port map( A3, B3, c3, S3, Cout);

end Behavioral;

Test bench:

library ieee;

use ieee.std_logic_1164.all;

entity sub_tb is

end sub_tb;

architecture behavior OF sub_tb is

COMPONENT sub

Port(

A3 : in STD_LOGIC;

A2 : in STD_LOGIC;

A1 : in STD_LOGIC;

A0 : in STD_LOGIC;

B3 : in STD_LOGIC;

B2 : in STD_LOGIC;

B1 : in STD_LOGIC;

B0 : in STD_LOGIC;

S3 : out STD_LOGIC;

S2 : out STD_LOGIC;

S1 : out STD_LOGIC;

S0 : out STD_LOGIC;

Cout : out STD_LOGIC);

END COMPONENT;

signal a3_tb : std_logic := '0';

signal a2_tb : std_logic := '0';

signal a1_tb : std_logic := '0';

signal a0_tb : std_logic := '0';

signal b3_tb : std_logic := '0';

signal b2_tb : std_logic := '0';

signal b1_tb : std_logic := '0';

signal b0_tb : std_logic := '0';

signal s3_tb : std_logic;

signal s2_tb : std_logic;

signal s1_tb : std_logic;

signal s0_tb : std_logic;

signal cout_tb : std_logic;

BEGIN

uut: sub PORT MAP (

A3 => a3_tb,

A2 => a2_tb,

A1 => a1_tb,

A0 => a0_tb,

B3 => b3_tb,

B2 => b2_tb,

B1 => b1_tb,

B0 => b0_tb,

S3 => s3_tb,

S2 => s2_tb,

S1 => s1_tb,

S0 => s0_tb,

Cout => cout_tb

);

process

begin

a3_tb <= '0'; a2_tb <= '0'; a1_tb <= '0'; a0_tb <= '0';

b3_tb <= '0'; b2_tb <= '0'; b1_tb <= '0'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '0'; b2_tb <= '0'; b1_tb <= '0'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '1'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '0'; a1_tb <= '0'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '1'; a1_tb <= '0'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '0'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '1'; a0_tb <= '0';

b3_tb <= '0'; b2_tb <= '1'; b1_tb <= '1'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '1'; a2_tb <= '1'; a1_tb <= '0'; a0_tb <= '0';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '0';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '0'; a1_tb <= '1'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

a3_tb <= '0'; a2_tb <= '1'; a1_tb <= '0'; a0_tb <= '1';

b3_tb <= '1'; b2_tb <= '0'; b1_tb <= '1'; b0_tb <= '1';

wait for 100 ns;

wait;

end process;

END;

Output:

3. 4-bit Multiplier

Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity Multiplier is

port(

A , B: in std_logic_vector(3 downto 0);

R: out std_logic_vector(7 downto 0)

);

end entity Multiplier;

architecture Behavioral of Multiplier is

begin

R <= std_logic_vector(unsigned(A) * unsigned(B));

end architecture Behavioral;

Test bench:

library ieee;

use ieee.std_logic_1164.all;

entity Multiplier_tb is

end Multiplier_tb;

architecture behavior OF Multiplier_tb is

COMPONENT Multiplier

port(

A , B: in std_logic_vector(3 downto 0);

R: out std_logic_vector(7 downto 0)

);

END COMPONENT;

signal A_tb : std_logic_vector(3 downto 0) := (others => '0');

signal B_tb : std_logic_vector(3 downto 0) := (others => '0');

signal R_tb : std_logic_vector(7 downto 0);

BEGIN

uut: Multiplier PORT MAP(

A => A_tb,

B => B_tb,

R => R_tb

);

process

begin

A_tb <= "0000";

B_tb <= "0000";

wait for 100 ns;

A_tb <= "1111";

B_tb <= "1111";

wait for 100 ns;

A_tb <= "1111";

B_tb <= "1111";

wait for 100 ns;

A_tb <= "1010";

B_tb <= "0101";

wait for 100 ns;

A_tb <= "1001";

B_tb <= "0110";

wait for 100 ns;

A_tb <= "1100";

B_tb <= "0110";

wait for 100 ns;

A_tb <= "0101";

B_tb <= "0110";

wait for 100 ns;

A_tb <= "0100";

B_tb <= "0101";

wait for 100 ns;

A_tb <= "1110";

B_tb <= "0110";

wait for 100 ns;

A_tb <= "0000";

B_tb <= "0110";

wait for 100 ns;

wait;

end process;

END;

Output:

The End